# Topology(WIP) and Vulnerabilities at 3DBioCAD

### What I have investigated and discovered so far (6/26/26)

It has only been 1 week since I started working at 3DBioCAD, and I have already discovered multiple vulnerabilities here at the company.

## Topology (WIP)

This is a work-in-progress version of the Complete network topology here at 3DBioCAD/B&B/KoDent/Arts Tech Group.

I will continue to iterate on this topology until the complete picture is present.

## Vulnerabilities

So far, I already have a few suggestions to shrink the attack surface of the company.

- Install Malwarebytes Antivirus

This Antivirus is a world leader in stopping Malware, Spyware, Trojans, and Ransomware in real-time. Due to a lot of the machines having Internet Access, having a robust Antivirus is Industry standard. In addition, since this office building houses a cluster of small businesses, I recommend purchasing the Small Business plan from Malwarebytes to enhance blanket endpoint security.

- Enable BitLocker and compile/print out recovery codes

Under normal circumstances, a properly configured small company network would include an “Active Directory/Domain Controller” for its ease of managing GPO (Group Policies), which could be deployed to all managed workstations at once via Policy Updates. However, I wasn’t present when the whole building was networked roughly 2 decades ago, so the current configuration is a vast network of decentralized local machines, each operating local Windows accounts. This configuration is fine, as our company does not consider internal security threats to be present, at least compared to external security threats (see RDP takeover/Assumed Lateral Movement Incident, which occurred in 2025).

Which brings us to why I recommend BitLocker and my proposed method for keeping BitLocker recovery keys. But first, let me explain what BitLocker is.

BitLocker is a built-in Windows security feature that protects data by encrypting the contents of a hard drive. Encryption converts files into unreadable data that can only be accessed with the correct credentials, helping protect sensitive information if a computer is lost, stolen, or an unauthorized person gains physical access to the device.

BitLocker works by automatically encrypting data stored on the drive and then decrypting it when an authorized user starts and uses the computer normally. It often works with a computer's security chip, called a Trusted Platform Module (TPM), to verify that the system has not been tampered with before allowing access to the encrypted data. To the user, BitLocker is largely transparent during everyday use while providing an additional layer of protection for stored information.

Normally, as I have mentioned earlier, under Industry Standard Security practices, an Active Directory/Domain Controller would escrow these recovery keys into a file, within an encrypted partition of a hard drive (Or just an Encrypted Hard Drive as a whole). Due to our lack of an AD/DC, I recommend printing out these recovery keys and keeping them easily accessible, but not visible, on my desk (Just like how Alex handed me the 6-digit Alphanumeric string Passwords to all the workstations). There is another possible issue with BitLocker Deployment; however, I am currently unsure if all the workstations within our cluster have TPM 2.0 enabled, which is a prerequisite for BitLocker to work. I would need more time to figure out whether each workstation has TPM 2.0.

- VLAN Segmentation (Requires Downtime)

To make the workplace even more secure, I could configure separate VLANs for different sections of the office. A VLAN is a Virtual Local Area Network in which the router artificially separates sections of the network so that they cannot communicate with each other. This boosts security to reduce the severity of a Lateral Movement Attempt/Attack if it were to ever occur again, by reducing the amount of surface area to which the attackers could see into the network. However, to implement this, I would require some network downtime, which, to my understanding, is not optimal for our work environment.

### Final Remarks

This concludes my first threat assessment after performing some passive reconnaissance into our network here at 3DBioCAD (& companies). There are more vulnerabilities and other suggestions that I might come up with later as I learn more about our network. Once again, thank you, Jack, for giving me this opportunity to work here as a Junior Security Consultant. I appreciate it more than I can express in words.

Jeffrey Zhang
Junior Security Consultant at 3DBioCAD
6/26/26